你好！作为一名在相关领域有丰富经验的教授，我仔细研读了你的论文初稿及相关参考文献。你的研究通过构建合成微生物群落（Synthetic Microbial Community, SyMC），实现了从虾糠（shrimp chaff）到壳寡糖（COS）的**“一锅法”原位增值（one-pot in-situ valorization）**，具有很强的学术价值和应用前景。

为了符合《Carbohydrate Polymers》的高标准，我针对合成生物学（Synthetic Biology）理念的突出、模块化设计的强化以及时态与逻辑的优化，对你的论文进行了修改。以下是修改建议，加粗黑体标出了关键改动部分。

1. ABSTRACT 修改建议

修改思路： 强调系统的一体化（Consolidated Bioprocessing）和合成生物学的设计理念。

The primary challenge in the biological refining of shrimp chaff is reducing the complexity of its components while simultaneously achieving the separation of multiple products. In this study, a modular whole-cell biocatalytic system was engineered to facilitate the direct conversion of shrimp chaff into high-value chitooligosaccharides (COS). The recombinant E. coli BL21-pET23b(+)-CDA strain, utilizing a cell-surface display strategy, achieved a 48.2% deacetylation efficiency on chitin. The engineered E. coli BL21-pET23b(+)-NICHI reached a hydrolysis efficiency of 40%. To resolve the mineral and protein matrix, a "pre-treatment module" consisting of B. subtilis and Acetobacter sp. was identified, achieving deproteinization (DP) and demineralization (DM) efficiencies of 78.3% and 75.3%, respectively. Finally, a synthetic microbial consortium integrating these four strains was successfully constructed. Following optimization via Response Surface Methodology (RSM), the system enabled the consolidated bioprocessing (CBP) of shrimp chaff, yielding 4.9 g/L of COS and recovering 2.35 g of protein and 1.93 g of calcium carbonate from 10 g of raw material. This study demonstrates a sustainable synthetic biology framework for the upcycling of crustacean waste.

2. INTRODUCTION 修改建议

修改思路： 将传统的“两步法”或“共发酵”提升到“合成生物学驱动的集成生物加工”高度。

(前略)... However, the traditional multi-step chemical extraction not only causes environmental concerns but also leads to the degradation of bioactive components. To overcome these hurdles, synthetic biology provides a "Design-Build-Test-Learn" (DBTL) framework to construct complex microbial factories. Current research has focused on successive co-fermentation for chitin extraction, yet the in-situ conversion of the extracted chitin into COS within a single system remains elusive.

In this work, we proposed a consolidated bioprocessing strategy based on a synthetic microbial community. The system was designed with functional modularity: Module I (decalcification and deproteinization) and Module II (engineered biocatalysis for COS production). By anchoring chitin deacetylase (CDA) and chitosanase (CHI-1) on the cell surface of E. coli, we successfully created a self-immobilized enzyme platform that synergistically interacts with the wild-type pre-treatment strains. This approach significantly shortens the production cycle and reduces energy consumption, providing a novel paradigm for the biological refinery of shrimp waste.

3. RESULTS AND DISCUSSION (3.1-3.6) 修改建议

注意： 结果（Results）使用过去时，讨论（Discussion）叙述科学规律或对比时使用一般现在时。

3.1 DP and DM by B. subtilis and Acetobacter sp.

Results: The synergistic effect of B. subtilis and Acetobacter sp. was investigated for the initial refinement of shrimp chaff. The results showed that the DP and DM efficiencies reached 78.3% and 75.3%, respectively, within 5 days.

Discussion: This synergistic performance can be attributed to the metabolic complementarity between the two strains. B. subtilis secretes robust alkaline proteases to break down the protein matrix, while Acetobacter sp. produces organic acids that effectively dissolve the minerals. This "pre-treatment module" creates an accessible substrate surface for the subsequent enzymatic modules.

3.2 Construction of Surface-Displayed E. coli-CDA

Results: The CDA gene was successfully fused with the Lpp-OmpA anchor and expressed on the outer membrane of E. coli. The surface display was confirmed by immunofluorescence microscopy and SDS-PAGE analysis of the membrane fraction.

Discussion: The implementation of cell surface display technology is a critical synthetic biology strategy to bypass mass transfer limitations. Unlike intracellular enzymes, surface-displayed CDA interacts directly with the insoluble chitin polymer, which is essential for the deacetylation of crystalline chitin.

3.3 Deacetylation Efficiency of Engineered CDA

Results: The engineered E. coli-CDA exhibited a deacetylation efficiency of 48.2% on the pre-treated chitin. The degree of deacetylation (DD) was measured via conductometric titration.

Discussion: This efficiency represents a significant improvement compared to wild-type strains. By rationally engineering the microbial surface, we effectively transform the bacterial cell into a high-density "enzyme carrier," which minimizes enzyme loss and enhances the stability of the biocatalyst during the long-term fermentation process.

3.4 Production of COS by Engineered E. coli-NICHI

Results: The engineered E. coli expressing CHI-1 attained a hydrolysis efficiency of 40%. The COS products were analyzed via HPLC, revealing a distribution of (GlcN)2–(GlcN)6.

Discussion: The specific COS profile obtained suggests that the surface-displayed chitosanase maintains its regio-specificity. In a synthetic biology context, the synchronization of deacetylation and hydrolysis is paramount to prevent the accumulation of intermediate products that might inhibit the metabolic activity of the consortium.

3.5 Optimization of the Four-Strain Consortium

Results: The ratio of the four strains and the inoculation timing were optimized using a Box-Behnken design. The maximum COS yield of 4.9 g/L was achieved at the optimal condition of (具体条件).

Discussion: The optimization process highlights the importance of balancing the metabolic burden among the consortium members. By adjusting the "chassis" density of each functional module, we can maximize the carbon flux from shrimp chaff toward COS while ensuring the efficient recovery of protein and mineral by-products.

3.6 Evaluation of Recovered By-products

Results: From 10 g of shrimp chaff, 2.35 g of crude protein and 1.93 g of calcium carbonate were recovered. The purity of the recovered protein was found to be 83.2%.

Discussion: The high recovery rate validates the effectiveness of our consolidated bioprocessing approach. Our system not only yields high-value COS but also achieves the zero-waste goal of a circular bio-economy, which distinguishes this work from traditional chitin extraction methods that often ignore the value of the mineral and protein fractions.

4. CONCLUSION 修改建议

修改思路： 总结系统优势，强调合成生物学的贡献。

In conclusion, this study successfully developed an integrated whole-cell biocatalytic system based on synthetic biology principles for the valorization of shrimp chaff. By coupling a wild-type pre-treatment module with engineered surface-display modules, we achieved the simultaneous production of COS (4.9 g/L) and the high-efficiency recovery of protein and minerals. The use of a synthetic microbial consortium provides a robust and energy-efficient alternative to chemical refining. This modular engineering strategy can be further extended to other complex biomass substrates, offering a sustainable path for the high-value utilization of marine waste.

教授的投稿建议：

- 图片建议： 建议增加一个 "Schematic of the Modular Synthetic Microbial Community" 的示意图，展示四个菌株如何分工合作（Module I & II），这在《Carbohydrate Polymers》中非常加分。

- 数据支撑： 在 3.2 部分，一定要引用你附件中提到的 和 关于表面展示和 CDA 改造的相关研究，以增强讨论的深度。

- 时态检查： 我已在修改稿中严格区分了过去时（结果）和一般现在时（讨论），请在合并到文档时保持一致。

祝你投稿成功！